The 2004 Volvo XC90

Safe vehicles don't have to be dull and boring

By Bill Siuru © AutoWire.Net

San Francisco: Ever since Volvo introduced the first three-point seatbelts in1959, this Swedish automaker has been an industry leader when it comes to safety. Therefore, when Volvo developed its first SUV, the XC90, safety was high on the priority list. However, Volvo showed that very safe vehicles don't have to be dull and boring.

For starters, there is an industry-first Roll Safety Control (RSC) system to reduce the chances of a rollover. The RSC system uses gyroscopic sensors to monitor the XC90's roll angle and roll velocity. If a possible rollover condition is sensed, the XC90's Dynamic Stability and Traction

Control (DTSC) system reduces engine power and applies brake until the SUV is stable again.

Should the RSC not prevent a rollover, the high-strength, boron-steel-reinforced roof protects occupants. There is also a reinforced steel cage around the passenger compartment. The XC90 has inflatable side curtain airbags for all three rows seats, another first. Pyrotechnic seat belt tensioners on all seats retain occupants in place so the front and side airbags can do their job.

The XC90 features crumple zones help absorb energy during a collision, even rear-end ones. Volvo's Whiplash Protection System reduces injuries when hit from the rear. The XC90 even looks out for other drivers since the low front chassis crossmember is about the same height as the bumper of an ordinary sedan.  Finally, there are daytime running lights, anti-lock brakes and large, high taillights to assure being seen under all lighting and weather conditions.

The XC90, marketed against other luxury SUV like the Acura MDX, BMW X5, Mercedes-Benz

M-Class and Lexus RX300 comes in three versions - the front-wheel-drive 2.5T and all-wheel-drive 2.5T AWD and T6 AWD. Two engines are available. The 2.5T uses a 2.5-liter turbocharged in-line five-cylinder and the T6, a 2.9-liter twin-turbocharged in-line-six. Both are mounted transversely. The T5 is rated at 208-horsepower at 5000 rpm and 236 ft-lb of torque from 1500 rpm to 4500 rpm. The T6 makes 268 horsepower at 5100 rpm and 280 ft-lb from 1800 rpm to 5000 rpm.

Both all-aluminum alloy engines feature a dual overhead camshaft (DOHC) driven by a belt, four-valves-per-cylinder and continuously variable valve timing (CVVT). The AWDs are driven by the front wheels, but as soon as slippage is detected, the electronically controlled Haldex limited slip coupling transfers from 5- to 100-percent of the torque to the rear wheels depending on road conditions.  Only two transmissions are offered, an automatic five-speed transmission with

Geartronic clutchless manual shifting for the T5. Because the five-speed wouldn't fit under the hood of the T6, it is mated to an automatic four-speed, also with Geartronic.

Like most full-size SUVs, the XC90 does not sip gasoline. The EPA ratings for the 2.5T are 18 mpg city/24 mpg highway and for the T6, 15 mpg city/20 mpg highway. The fuel tank holds 19 gallons and the XC90 can tow 5000 pounds when properly equipped.

Like Volvo wagons, the interior is both cavernous and versatile. According to Volvo, the seats can be arranged in over 64 configurations. For example, the second- and optional third-row seatbacks fold flat. The front passenger seatback folds down so there are nine-and-a-half feet of unobstructed space for very long items. The second-row seats are split 40/20/40 and slide forward independently and headrests do not have to be removed when seats are folded down. With the forward-facing third-row seat installed, the XC90 can carry up to seven. An optional center-child-seat booster slides forward, placing it closer to the front passenger seat. The tailgate features a 70/30 top/bottom split so the lower edge of the liftgate is waist high. Unless you are loading a light object, you probably will have to open both sections.

On the road, the XC90 shows it unit-body, passenger car heritage. It is based on Volvo's largest car platform, the S80. Therefore, the XC90 rides more like a luxury sedan than many of its truck-based SUV competition. And while not a sports sedan, it handles, steers and brakes like a very good family station wagon; remember that it can carry up to seven. It was especially adept at smoothing out parking lot speed bumps. While most XC-90s will never see much off-road duty, it is quite capable since it is a SUV not a station wagon posing as an SUV. The tight turning radius is appreciated in both city traffic and off-road. About the only compliant is a bit of turbo lag when accelerating.

Climbing behind the comfortable and very supportive driver seat, you find an expansive dashboard. Though easy to read, instrumentation is limited to speedometer, tachometer and gauges for fuel and coolant temperature. Controls for the audio and climate control systems are very user-friendly, but the display is hard to read under some daylight conditions.

Prices for the XC90 start at $35,125 for the 2.5T FWD, $38.875 for the 2.5T AWD and $41,250 for the T6.  Even the lowest priced XC90 comes well equipped. However, there is a long list of options including an onboard navigation system ($1895), reverse warning system ($400), child booster seat ($400), Dolby Pro Logic II Surround Sound System ($750), Xenon headlamps ($300), vertical cargo net ($300), wood steering wheel ($300), and rear audio controls with headphones ($100).

By Bill Siuru © AutoWire.Net - San Francisco.

Byline:  Syndicated content provided by Tony Leopardo © AutoWire.Net

Column Name: Safe vehicles don't have to be dull and boring

Topic:  The 2004 Volvo XC90

Word Count:  940

Photo Caption:  The 2004 Volvo XC90

Photo Credits:  Volvo Internet Media

Series #:   2003 - 46